West Gippsland Waterwatch Volunteer Water Quality Monitoring Data Sheet

Site Details Site Code: MOR040 Site Name: Morwell River @ Nuttalls rd Bridge Site Grid Reference: Easting: Northing:

Test Details Date: 13/01/2019 Person(s) Conducting the Survey / Test: Joelle Champert Time: AM PM 11.15 Name of Monitoring Group: Yinnar-Yinnar South Landcare

Your Meter - Reading Before Calibration (when in CAL Mode) EC Meter: 1310 pH Meter in 7.01: 7.3 pH Meter in 4.01: 4.1

Your Meter - Reading Post Calibration (after pressing the ENT button) EC Meter: 1410 pH Meter in 7.01: 7.0 pH Meter in 4.01: 4.0

Test Results Electrical Conductivity: 310 μS mS Dissolved Oxygen: mg/L % Reactive Phosphorus: 0.03 mg/L P Air Temperature: 26 oC Water Temperature: 20.2 oC pH: 7 Turbidity: 17 N.T.U.

Weather Conditions at Time of Sampling (please tick) Sunny Cloudy Overcast Raining Windy

Last Rainfall (please tick) More than a week ago During the last week During the last 24 hrs Raining Now Amount of rain: 3 mm

Flow Indicator: ML/day

Water Flow: (please tick) Flood/Overbank - Flow is high and water level has exceeded bank capacity and is connecting with floodplain Bankfull - High flow within channel capacity and in-stream habitats connected High - Less than bankfull and in-stream habitats connected Medium - Flow is normal / typical average flow Low - Minimum flow in channel/ continuous flow in some part of channel (in-stream habitat connected) Dry - No continuous flow in channel and no pools Pools - No continious flow in channel and intermittent pools (in-stream habitats not connected) Stagnant - No continuous flow in channel but channel continuous (instream habitats connected) Permanent - Wetlands and lakes

Water Appearance (please tick) Clear Scummy Smelly Foamy / Frothy Stained Brown Muddy Milky Oily Stained Green Other:

Stream Depth and Width (please tick) Depth: 0-50cm 51-99cm 1-2m >2m Other: Width: 2-5m >5m Unknown Other:

Drain Flow (please tick) flowing not flowing Colour: Odour:

Circumstantial Hazards and Additional Risks Hazard: Risk: Risk Control Measures:

Litter Pollutants (please tick) Bottles Car bodies Oil Paper Plastic Waxed cardboard Cans Clothing Packets Petrol/Diesel Polystyrene Other:

Comment: